nRF5340 can absolutely be used for two-way voice communication, but there is an important distinction between using another nRF5340/BLE Audio device and using a generic “normal Bluetooth module.”

### 1. If you mean this architecture

Microphone

↓

ADC / PDM

↓

nRF5340

↓

LC3 codec

↓

Bluetooth LE Audio

))))))))))))))

((((((((((((((

Bluetooth

↓

nRF5340

↓

DAC / I2S

↓

Speaker

Then yes — this is a good application for nRF5340. Nordic specifically provides LE Audio examples for nRF5340, including unicast audio and headset-type applications. The nRF5340 supports the LC3 codec through the nRF Connect SDK.

If by “normal Bluetooth module” you mean something like an HC-05/HC-06 Bluetooth Classic module, then not directly for the same LE Audio implementation. Those modules use Bluetooth Classic profiles such as SPP and aren't suitable as an LC3/Bluetooth LE Audio endpoint.

## 2. For a voice communication device, I'd recommend this

For example, a wireless intercom:

DEVICE A                         DEVICE B

Mic ──> nRF5340                 nRF5340 <── Mic

│                         │

│   Bluetooth LE Audio    │

└─────────────────────────┘

│                         │

Speaker <─┘                         └─> Speaker

Both sides can transmit and receive voice.

The nRF5340 is particularly suitable because it has two Cortex-M33 cores: the application core handles the application/audio processing while the network core handles Bluetooth LE.

# 3. Hardware you need

For a prototype, I would use:

### Option A — easiest

2 × nRF5340 Audio DK

It already has the audio hardware, including the Cirrus Logic CS47L63 audio DSP and audio connectors. Nordic recommends the nRF5340 Audio DK for LE Audio development.

DEVICE A

┌──────────────────┐

Mic ───>│                  │

│   nRF5340        │

│                  │

│   LE Audio       │

└────────┬─────────┘

│

│ BLE

│

┌────────▼─────────┐

│   nRF5340       │

│                  │

│   LE Audio       │

└────────┬─────────┘

│

Speaker

This is the route I recommend if your goal is to develop the software first and later make your own PCB.

# 4. Can you develop completely offline?

Yes, after you download the required SDK/toolchain/packages.

The normal development environment is:

- VS Code

- nRF Connect SDK

- Zephyr

- ARM toolchain

- west

- nRF Command Line Tools

### Easier method

You can also get the executable through Nordic's GitHub/package infrastructure:

Nordic nRF Util package index on GitHub

After downloading/extracting:

nrfutil.exe

put it somewhere such as:

C:\nrfutil\

Then add:

C:\nrfutil

to your Windows PATH.

Open a new Command Prompt and test:

nrfutil --version

You should get something similar to:

nrfutil 7.7.1

### Then install the device command

For your nRF5340 development:

nrfutil install device

Then:

nrfutil device list

With your nRF5340 DK connected, this should show the connected Nordic device.

- J-Link / programmer

Nordic's nRF Connect SDK includes Zephyr, Bluetooth stacks, drivers, samples and LE Audio support.

Once you have everything downloaded, you can work without Internet.

### Recommended offline setup

On an Internet-connected PC first:

nRF Connect SDK

↓

Zephyr

↓

Toolchain

↓

Bluetooth / LE Audio libraries

↓

LC3 library

↓

Samples

Then copy the complete SDK/toolchain installation to your offline development PC.

# 5. You don't need to write the entire Bluetooth stack yourself

This is important.

You shouldn't start by writing raw Bluetooth voice communication code.

Nordic already provides:

Bluetooth Controller

+

Bluetooth Host

+

ISO channels

+

LE Audio

+

LC3 codec

The LC3 codec is provided as a software library in nRF Connect SDK.

There are also existing BAP Unicast Client/Server samples for nRF5340.

# 6. Example starting code

For example, your application could eventually have something like:

#include <zephyr/kernel.h>

#include <zephyr/bluetooth/bluetooth.h>

#include <zephyr/bluetooth/conn.h>

static void connected(struct bt_conn *conn, uint8_t err)

{

if (err) {

printk("Connection failed: %u\n", err);

return;

}

printk("Bluetooth connected\n");

}

static void disconnected(struct bt_conn *conn, uint8_t reason)

{

printk("Bluetooth disconnected: %u\n", reason);

}

BT_CONN_CB_DEFINE(conn_callbacks) = {

.connected = connected,

.disconnected = disconnected,

};

int main(void)

{

int err;

printk("Voice device starting...\n");

err = bt_enable(NULL);

if (err) {

printk("Bluetooth initialization failed: %d\n", err);

return 0;

}

printk("Bluetooth initialized\n");

while (1) {

k_sleep(K_SECONDS(1));

}

return 0;

}

But this is only the Bluetooth foundation.

It does not yet transmit voice.

For actual voice you need:

Microphone

↓

PCM samples

↓

LC3 encoder

↓

ISO audio packets

↓

BLE

↓

ISO audio packets

↓

LC3 decoder

↓

PCM samples

↓

Speaker

# 7. The actual voice software architecture

I'd structure your project like this:

voice_device/

│

├── CMakeLists.txt

├── prj.conf

├── boards/

│

└── src/

│

├── main.c

│

├── bluetooth_audio.c

├── bluetooth_audio.h

│

├── microphone.c

├── microphone.h

│

├── speaker.c

├── speaker.h

│

├── lc3_codec.c

└── lc3_codec.h

And:

main.c

│

┌────────┴────────┐

│                 │

Bluetooth           Audio

│                 │

│          ┌──────┴──────┐

│          │             │

│       microphone    speaker

│          │             ▲

│          ▼             │

│       PCM audio        │

│          │             │

│       LC3 encode       │

│          │             │

▼          ▼             │

ISO/BLE ───────────────> LC3 decode

# 8. There is an even better way to start

Rather than writing this from zero, start from Nordic's existing BAP Unicast Audio Client/Server example.

For nRF5340, Nordic documents building the application with:

west build -b nrf5340dk/nrf5340/cpuapp --sysbuild samples/bluetooth/audio/bap_unicast_client/

The corresponding server sample can be used for the other side.

This gives you a much better starting point than trying to implement the entire LE Audio protocol yourself.

## 9. If you want a walkie-talkie

If your actual goal is something like:

PERSON A

🎤

│

nRF5340

│

Bluetooth

│

▼

nRF5340

│

🔊

PERSON B

with push-to-talk, we can make it significantly simpler.

For example:

Button pressed

Button

↓

Start microphone

↓

PCM

↓

LC3

↓

BLE ISO

↓

Other nRF5340

↓

LC3

↓

Speaker

Release button:

Stop transmission

For full duplex:

Mic A ───────────────> Speaker B

Mic B ───────────────> Speaker A

That is also possible.

## 10. One important hardware question

If you are using a bare nRF5340 module, you need to provide the audio interface yourself.

For example:

nRF5340

┌─────────┐

PDM MIC ─────>│         │

│         │

I2S DAC <─────│         │

│         │

BLE antenna <─│         │

└─────────┘

Whereas the nRF5340 Audio DK already provides the audio-related hardware, which is why it is much easier for development.

### My recommendation for your project

Start with:

2 × nRF5340 Audio DK → get two-way voice working → move the software to your nRF5340 module → design your custom PCB.

Absolutely. I can give you the complete starter project and offline setup method.

I recommend building it in two stages:

- First: get two nRF5340 boards communicating with Bluetooth.

- Then: add microphone → LC3 → Bluetooth ISO → LC3 → speaker for two-way voice.

For the first prototype, an nRF5340 Audio DK is much easier than a bare module because the microphone/audio hardware is already available.

If you have a specific nRF5340 module or development board, you can upload a photo of it; I can identify the board/module and adapt the wiring and project configuration to it.

### Offline development setup

On your Windows PC, install/download these while you have Internet:

1. VS Code

2. nRF Connect for Desktop

3. nRF Connect SDK

4. nRF Connect SDK Toolchain

5. nRF Command Line Tools

6. SEGGER J-Link software

Then your offline project can look like:

C:\nrf\

│

├── ncs\

│   ├── zephyr\

│   ├── nrf\

│   ├── nrfxlib\

│   └── modules\

│

└── voice_project\

├── CMakeLists.txt

├── prj.conf

└── src\

├── main.c

├── audio.c

├── audio.h

├── microphone.c

├── microphone.h

├── speaker.c

└── speaker.h

### Build command

Once the SDK is installed:

cd C:\nrf\voice_project

west build -b nrf5340dk/nrf5340/cpuapp --sysbuild

Flash:

west flash

View debugging output:

west debug

### Voice pipeline

The final application will be:

nRF5340 #1

┌─────────────────────┐

│                     │

MIC ──>│ PDM                 │

│   ↓                 │

│ PCM                 │

│   ↓                 │

│ LC3 ENCODER         │

│   ↓                 │

│ Bluetooth ISO       │

└─────────┬───────────┘

│

│  BLE Audio

│

┌─────────▼───────────┐

│ Bluetooth ISO       │

│   ↓                 │

│ LC3 DECODER         │

│   ↓                 │

│ PCM                 │

│   ↓                 │

│ I2S / Audio Codec   │

└─────────┬───────────┘

│

SPEAKER

And simultaneously the reverse direction operates:

MIC #2 → LC3 → BLE → LC3 → SPEAKER #1

so you get full-duplex voice communication.

### Important

I would not recommend starting with an HC-05/HC-06 "normal Bluetooth module." For this project, use two nRF5340 devices and Bluetooth LE Audio. If you specifically need to communicate with an existing Bluetooth Classic headset/module, that's a different architecture.

Below is a real starter code set for nRF5340 + Zephyr/nRF Connect SDK. One important point: a complete production Bluetooth LE Audio voice system is much larger than a few C files, so this first version gives you the Bluetooth connection + audio application structure you can build on.

If you upload a photo of your exact nRF5340 board/module, I can adapt the pin configuration for your hardware.

## 1. Project structure

Create:

voice_nrf5340/

├── CMakeLists.txt

├── prj.conf

└── src/

└── main.c

## 2. CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0)

find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE})

project(voice_nrf5340)

target_sources(app PRIVATE

src/main.c

)

## 3. prj.conf

Start with this:

CONFIG_BT=y

CONFIG_BT_PERIPHERAL=y

CONFIG_BT_CENTRAL=y

CONFIG_BT_DEVICE_NAME="NRF5340_VOICE"

CONFIG_BT_MAX_CONN=2

CONFIG_GPIO=y

CONFIG_SERIAL=y

CONFIG_CONSOLE=y

CONFIG_UART_CONSOLE=y

CONFIG_PRINTK=y

CONFIG_MAIN_STACK_SIZE=4096

CONFIG_SYSTEM_WORKQUEUE_STACK_SIZE=4096

This is the initial Bluetooth/application foundation.

# 4. main.c

#include <zephyr/kernel.h>

#include <zephyr/device.h>

#include <zephyr/sys/printk.h>

#include <zephyr/bluetooth/bluetooth.h>

#include <zephyr/bluetooth/conn.h>

#include <zephyr/bluetooth/gatt.h>

#include <zephyr/bluetooth/hci.h>

static struct bt_conn *current_conn;

/* ---------------------------------------------------------

* Bluetooth connection callbacks

* --------------------------------------------------------- */

static void connected(struct bt_conn *conn, uint8_t err)

{

char addr[BT_ADDR_LE_STR_LEN];

bt_addr_le_to_str(bt_conn_get_dst(conn), addr, sizeof(addr));

if (err) {

printk("Connection failed: %s (%u)\n", addr, err);

return;

}

printk("Connected: %s\n", addr);

current_conn = bt_conn_ref(conn);

}

static void disconnected(struct bt_conn *conn, uint8_t reason)

{

char addr[BT_ADDR_LE_STR_LEN];

bt_addr_le_to_str(bt_conn_get_dst(conn), addr, sizeof(addr));

printk("Disconnected: %s reason=%u\n", addr, reason);

if (current_conn) {

bt_conn_unref(current_conn);

current_conn = NULL;

}

}

BT_CONN_CB_DEFINE(conn_callbacks) = {

.connected = connected,

.disconnected = disconnected,

};

/* ---------------------------------------------------------

* Advertising

* --------------------------------------------------------- */

static const struct bt_data ad[] = {

BT_DATA_BYTES(BT_DATA_FLAGS,

(BT_LE_AD_GENERAL |

BT_LE_AD_NO_BREDR)),

BT_DATA(BT_DATA_NAME_COMPLETE,

CONFIG_BT_DEVICE_NAME,

sizeof(CONFIG_BT_DEVICE_NAME) - 1),

};

/* ---------------------------------------------------------

* Start Bluetooth

* --------------------------------------------------------- */

static int bluetooth_start(void)

{

int err;

printk("Starting Bluetooth...\n");

err = bt_enable(NULL);

if (err) {

printk("Bluetooth initialization failed: %d\n", err);

return err;

}

printk("Bluetooth initialized\n");

err = bt_le_adv_start(

BT_LE_ADV_CONN,

ad,

ARRAY_SIZE(ad),

NULL,

0

);

if (err) {

printk("Advertising failed: %d\n", err);

return err;

}

printk("Advertising started\n");

return 0;

}

/* ---------------------------------------------------------

* Audio initialization

* --------------------------------------------------------- */

static int audio_init(void)

{

printk("Initializing audio system...\n");

/*

* Later:

*

* PDM microphone

*       ↓

* PCM samples

*       ↓

* LC3 encoder

*       ↓

* Bluetooth ISO

*/

return 0;

}

/* ---------------------------------------------------------

* Main

* --------------------------------------------------------- */

int main(void)

{

int err;

printk("\n");

printk("==============================\n");

printk(" nRF5340 Voice Device\n");

printk("==============================\n");

err = audio_init();

if (err) {

printk("Audio initialization failed\n");

return 0;

}

err = bluetooth_start();

if (err) {

printk("Bluetooth start failed\n");

return 0;

}

while (1) {

if (current_conn) {

printk("Bluetooth connected - voice system ready\n");

} else {

printk("Waiting for Bluetooth connection...\n");

}

k_sleep(K_SECONDS(5));

}

return 0;

}

# 5. Build it

For the Nordic nRF5340 DK:

cd C:\nrf\voice_nrf5340

Then:

west build -b nrf5340dk/nrf5340/cpuapp --sysbuild

Flash:

west flash

Open the serial terminal and you should see:

==============================

nRF5340 Voice Device

==============================

Initializing audio system...

Starting Bluetooth...

Bluetooth initialized

Advertising started

Waiting for Bluetooth connection...

# 6. But this isn't voice yet

The next important code is the audio path.

You ultimately want:

TRANSMIT

Microphone

│

▼

PDM driver

│

▼

PCM 16 kHz

│

▼

LC3 encoder

│

▼

ISO packet

│

▼

Bluetooth

And:

RECEIVE

Bluetooth

│

▼

ISO packet

│

▼

LC3 decoder

│

▼

PCM

│

▼

I2S

│

▼

Speaker

For actual LE Audio, I recommend that we don't manually implement the LC3/ISO layer on top of the simple Bluetooth example above. Instead, use Nordic's existing BAP Unicast Client/Server samples and modify their audio application. That gives you the correct LE Audio architecture rather than creating a custom incompatible protocol.